UPCOMING MATCH #: _____________     BUMPER COLOR FOR MATCH (CIRCLE ONE):     RED      BLUE

| IN PIT (ROBOT OFF) – NOTE REPAIRS |  |
| --- | --- |
| ❏ Change bumpers for upcoming match. Attached well? ❏ Plug in Driver Console laptop to charge. ❏ Check for loose parts/motors/gearboxes/wires/hoses ❏ Check battery cable connections to PDP & breaker ❏ Any dangling or damaged wires? | ❏ Check screws/connections on subsystems: - DRIVETRAIN – Plates, motors, wheels - INTAKE – Shafts, standoffs, brackets - LAUNCHER/PLACER – Shafts, structure - CLIMBER – Bolts, structure, shafts |

| NON-POWERED SUBSYSTEM CHECKS – NOTE REPAIRS |  |
| --- | --- |
| ❏ DRIVETRAIN | Rotation / Steering / Power transmission / Clean? |
| ❏ INTAKE | Shaft collars tight / Belts tight and aligned / Structural alignment Deployment moves correctly / Check motor and/or piston mounts |
| ❏ LAUNCHER/PLACER | Structural alignment / Belts to motors properly attached / Wheels/rollers spin smoothly / Aiming system |
| ❏ CLIMBER | Structure (wear or stress?) Hooks straight? / Strap wound properly, ratchet in correct position |
| ❏ ELECTRICAL | Unmanaged or dangling wires / Check wires near moving or sharp parts for wear / CAN wire check / Components securely mounted |
| ❏ BUMPERS | Correct color / Brackets intact and firm / Missing or loose hardware |

| IN PIT (ROBOT ON) - NOTE ANY CODE OR MECHANICAL CHANGES |  |
| --- | --- |
| What changed? | Notes: |

| ENABLE ROBOT: POWERED SUBSYSTEM FUNCTIONAL CHECKS – NOTE REPAIRS |  |
| --- | --- |
| ❏ Electronic Components | RSL: Blinking orange Camera(s): Feed(s) working on dashboard Radio: Securely mounted / Power and data cables / Connection |
| ❏ Drivetrain (wheels off the ground!) | Drive wheels Forward / Back / Turning / Properly aligned? |
| ❏ Intake | Deploy / Retract / Run intake / Run outtake / Reset for match |
| ❏ Launcher/Placer | Loading / Run up to speed / check aiming |
| ❏ Climber | Full extension / Retraction / Reset for match |
| ❏ Pneumatics: | Release Valve Closed / Holding Pressure / Watch gauge for leaks Pressure: ❏ 60psi working ❏ 120psi stored |

| ROBOT OFF (preparing to queue): |  |
| --- | --- |
| ❏ Install New Battery #_________________________ ❏ Secure connector and Tug Test. Connector ok? Battery Beak: ❏ Charge: ____________ ❏ Status: _____________ | ❏ Pack Driver Station with Laptop, controllers, charger ❏ Pack Robot Cart with necessary items: ❏ Tools ❏ Team Flag ❏ Spare Battery ❏ Gaffer’s Tape ❏ USB Tether Cable |